Release Notes

| Program: L6260 | Revision Number: 6.11.1211’15 Replaced Revision: 6.11.0417’15 Creation Date: 12/11/2015 |  |  |
| --- | --- | --- | --- |
| Author or Designer: Peishan |  |  |  |
| What Changed? |  |  |  |
| What has been Fixed? |  | Avscan_n.dll - Delete avbarcode.in in the barcode initialization. - Fixed the wrong barcode type output when specifying barcode. - Support the length of barcode to 1024 bytes. - Fixed the halftone is not working when the scanning data is color. - Modify for SmartImage algorithm ehancement. - Be able to set the source buffer to the destination in DoJpegCrop(). - Define ""LongPageMemory"" in ini for long page memory usage." - Support to convert raw data with jpeg cropping in long page mode. - Reallocate the memory when allocating memory failure in CreateDIB(). - The memory usage log supports over 4GB. - To specify the long page transportation mode is file mapping or memory. - Fixed the multiple scan area problem in auto orientation mode. - Fixed the driver is freezed when blank page removal in page scan mode. - Moves the punch hole removal in the front of blank page removal. - Fixed the background value error for blank page removal. - Modify for accusoft barcode library supported. - Fixed the wrong multifeed information in old AVAPI. - Bug fixed for IR and UV scan. - Support IR and UV scan. - Modification for troubleshooting on paper jam or cover open. - Continue to scan after troubleshooting. - Fixed one more page scanning in page scan mode. - Flatbed cropping by SW. - Support to adjust the cutting size in EOF scanning. - Fixed the scanning speed drop down. - Send the gamma table to VM2 device when the image is 600 dpi without any image processing. - Fixed the flatbed scanning in NewAVAPi. - Fixed the program hang up when the scanning thread is finish but transfer thread is running. - Show long page cropping failed message when scanning all pages done. - Support flatbed cropping mode in NewAvapi." - Support the long page cropping by SW in NewAvapi. - Flip the front side in merge two sides without carrier sheet. - Apply the color invert setting for all output image in auto color detection mode. - Handle the scanner error from stop scanning in new AVAPI. L6260.ds - Support to eject paper backward direction in NewAvapi. - Fixed inspector TWAIN failure in CAP_DUPLEXENABLED with simplex scanner. - Support the length of barcode to 1024 bytes. - To discard long page option in MICR connected, it still exists in imprinter connected without MICR. - Modify for Korean SKT project. - Modify for China Construction Bank application. - Modify for accusoft barcode library supported. - Fixed the return wrong value for ICAP_PHYSICALWIDTH and ICAP_PHYSICALHEIGHT in HandleGetCap(). and HandleGetCurrentCap(). - Modify for dual interface scanners in NewAvapi. - Support to write whole nvram data in NewAvapi. - Support virtul scanner protocol. - Replace the no paper error with device error in EnableDS(). - Support Infrared(IR) and Ultraviolet(UV) setting. - Show adf no paper when the scan source is automatic and there is no front side image selected. - Fixed the lineart image output only in non-UI mode when there is no front image selected. - Modify for troubleshooting behavior. - Support to hide the waiting paper feed dialog in INI. - Fixed the wrong default scan length in flatbed on non-UI. - Support to continue scan after troubleshooting. - Fixed the incorrect area while automatic scan source with flatbed. - Fixed the wrong behavior while transport timeout is on. - Support SW control prefeed time in NewAVAPI. - Behavior changes for ICAP_COMPRESSION. - Support Long page cropping with SW for NewAvapi. - Windows 10 can be able to recognize. - Modify for merge two sides patent is expired. - Change the reverse roller life to 100000. - Fixed the power off is unchecked problem. Twdsm_n.dll - Handle the scanner error in preview scan. Advance.dll - Modify eject paper opposite for newapi - Modify x64 press enter to scan. - Modify the minimum adjustment - Modify overscan right always be disable in flatbed for avapi. - Support A3 size to merage two side - Modify refresh or redraw may flashing - Modify longpage minimum length - Add a barcode type dialog - Modify get dual device name. - Modify NewWriteNVMData parameter - Add hotkey to open directory(C:\ProgramData\MODELNAME) - Modify paper tab set landscape may invalid, tab menu check status. - Get OS name from registry - Modify user defined size may less than minimum size - If select infrared or ultraviolet will uncheck rear's options. - Add infrared and ultraviolet,modify rear setting on scan - Modify preview area may be wrong ,Modify rear setting on preview scan - Russian correction - When switching tab,set keyboard focus on scan button - Modify non-ui mode to be not call SetPowerSaving - Set keyboard focus on scan button - Modify background color maybe wrong display in information tab, when page scan is disable that will not be changed. - Add SideTruncateMax to define minimum overscan - Modify setting landscape may invalid,Fixed change to paper tab may shut down - Modify minimum length and width will reference FlatbedMinLength and ADFMinLength. - Modify property sheet has three rows, button will be cut - Modify default fixed size display wrong - Modify may exceed scan area - Modify deskew edit length,default fixed size display wrong - Set long paper length when select resolution(last set less than limit length) . - Add merge two side option (England,United States,France,Germany) - Modify Windows 10 version text in information tab. |  |
| Known Defects |  |  |  |
| Required Files And Revision Level |  | File Name | Revision |
|  |  | Avscan_n.dll | 3.0.1.46 |
|  |  | L6260.ds | 3.0.1.40 |
|  |  | Twdsm_n.dll | 3.0.1.8 |
|  |  | Advance.dll | 3.1.1.68 |
|  |  | L6260.dll | 0.9.2015.410 |
|  |  | SmartImage.dll | 1.1.107.0 |
|  |  | AVIP.dll | 0.0.0.50 |
|  |  | AVJpegEncoder.dll | 1.1.0.4 |
|  |  | AVScanDoubleSizeLib.dll | 1.3.0.1 |
|  |  | Cmatch.dll | 4.3.9.4 |
|  |  | AVBarcode.dll | 1.0.0.8 |
|  |  | Av2PDF.dll | 1.0.0.6 |
|  |  | AVBlank.dll | 0.1.0.3 |
|  |  | fpcreator_img.dll | 1.0.0.617 |
|  |  | AvCropEx.dll | 0.5.10.0 |
|  |  | Gray_quality.dll | 1.0.0.5 |
|  |  | LDyIp.dll | 2.0.1.11 |
|  |  | AVglowing.dll | 1.0.49.0 |
|  |  | ColorDropOut.dll | 2005/08/26 |
| Details: |  |  |  |